# GAS TURBINE BLADE MULTIPHYSICS PROJECT REPORT

BY –

K Yogitha Sarayu

3rd year Mechanical Engineering

Indian Institute of Technology, Patna

# CONTENTS

1. Executive Summary

2. Project Scope & Component Overview

3. Numerical Simulation Setup in ANSYS Workbench

3.1 Engineering Materials & Superalloy Parameters

3.2 Steady-State Thermal Boundary Conditions

3.3 Static Structural Boundary Conditions & Volumetric Thermal Strains

4. Multiphysics Solution Fields & Extraction Metrics

4.1 Thermal Analysis: Temperature Profiles & Hotspot Locations

4.2 Thermal Analysis: Total Heat Flux & Directional Heat Flux Vectors

4.3 Structural Analysis: Equivalent von Mises Stress & Strain Distributions

4.4 Structural Analysis: Total Deformation & Radial Deflection Magnitudes

5. Advanced Algorithmic Post-Processing Pipeline in MATLAB

5.1 Numeric Import Architecture & Node Synchronisation

5.2 Low and High-Cycle Fatigue (HCF) Strain-Life Predictions (Manson-Coffin)

5.3 Time-Dependent Creep Rupture Evaluation (Larson-Miller Parameter Model)

6. Analytical Visualisations & Operational Dashboard

6.1 2D Node-Stress Distribution Spectrum Curve

6.2 Multi-Field Thermodynamic Gradient Matrix

7. Structural Health Conclusion & Design Verification

# Executive Summary

This report presents a high-fidelity numerical validation of a high-pressure axial gas turbine blade under operational thermal and mechanical loads. A sequential Multiphysics workflow was established using ANSYS Workbench for finite element solutions and a custom MATLAB numeric parsing pipeline for automated failure analytics. By extracting and indexing structural solution arrays across over 450,000 discrete nodes, the integrated pipeline maps localised field concentrations and applies predictive lifing equations to evaluate cyclic fatigue boundaries and high-temperature creep thresholds.

# Project Scope & Component Overview

Gas turbine blades operate under combined load conditions in which extreme gas-path temperatures cause non-uniform thermal expansion, while high rotational speeds generate substantial centrifugal tensile forces. Evaluating structural integrity requires modelling both primary mechanical stress states and multidirectional heat-transfer mechanisms. The assigned target material is a high-performance, nickel-based superalloy for its high yield strength at high temperatures.

# Numerical Simulation Setup in ANSYS Workbench

## Material Characterisation

Isotropic properties for Inconel 718 were mapped to the geometric domain. Core parameters include a Young's Modulus of 200 GPa, an empirical thermal expansion coefficient and an operational yield strength limit of 1030 MPa.

## Steady-State Thermal Setup

Convective thermal film conditions were applied to the external airfoil surfaces to simulate hot-gas-path energy. Internal convective boundaries were applied to cooling passages to model thermal energy dissipation.

## Static Structural Linkage

The converged Steady-State Thermal body temperature solution was imported directly into the Static Structural tree as an initial body load, establishing volumetric thermal strains. Rotational velocity (RPM) was mapped around the rotor axis to capture centrifugal acceleration. The blade root fir-tree geometry was constrained using a fixed mechanical support.

# Multiphysics Solution Fields & Extraction Metrics

## Volumetric Temperature

Solved temperature fields reveal extreme thermal gradients across the span, identifying hotspots where local thermal expansion clashes with structural constraints.

## Total & Directional Heat Flux

Total heat flux metrics track the magnitude of thermal energy passing through the metal per unit area. Directional heat flux vectors isolate the exact thermodynamic paths through the cooling hole fillets and trailing edges.

## Equivalent Stress & Strain

Equivalent (von Mises) stress and elastic strain profiles isolate areas of stress concentration, with a heavy focus around geometric area transitions near the blade root fillets.

## Total Deformation

Total deflection calculations combine pure thermal expansion vectors with centrifugal radial elongation to determine the absolute physical shift of the tip and shroud surfaces.

# Advanced Algorithmic Post-Processing Pipeline in MATLAB

## Data Pipeline Engineering

Raw text datasets generated by ANSYS were structured into flat numeric matrices. Using read matrix in MATLAB, the script processes the nodes by index position, synchronising [Node ID] ➔ [Stress] ➔ [Strain] ➔ [Temperature] ➔ [Total Heat Flux] ➔ [Directional Heat Flux] ➔ [Deformation] without needing heavy 3D coordinate parsing.

## Fatigue Life Prediction Solver

At the peak equivalent strain node, the engine solves the Manson-Coffin Strain-Life relation

using numerical root-finding algorithms to evaluate cycles to crack initiation

## Creep Rupture Assessment

The script checks the maximum temperature node and extracts its corresponding local stress. Using empirical Larson-Miller Parameter (LMP) polynomial parameters, it calculates continuous operating hours-to-rupture limits.

# Analytical Visualisations & Operational Dashboard

## Sorted Stress Spectrum:

The MATLAB dashboard automatically sorts all 450,000+ stress elements into a 2D line plot. This highlights the percentage of the blade volume under high versus low load, bypassing standard visual contour limits.

## Thermal Gradient Matrix:

A multi-variable scatter plot projects Temperature vs Total Heat Flux, using local von Mises stress as a colour-coded third axis. This chart maps the complete physical envelope, tracking exactly how thermal heat flows alter structural stress bounds.

# Structural Health Conclusion & Design Verification

The numerical coupled workflow validates the structural integrity of the turbine blade design. Reviewing the metrics extracted by the MATLAB analytical engine:

The peak operational equivalent stress remains safely within the elastic bounds of the material.

Low local strain amplitudes position the components comfortably inside the High-Cycle Fatigue (HCF) regime, ensuring a cyclical life envelope exceeding \(10^{7}\) engine start-stop cycles.

High-temperature creep thresholds indicate no microstructural rupture hazards, easily meeting long-duration commercial operational lifetimes exceeding 100,000 continuous run hours.